Requirment specification

| Project Title | Wetterstation |
| --- | --- |
| Project Leader | Matthias Brückl |
| Created on | 03.12.2025 |
| Last changed on | 25.01.2026 |
| state | Currently in work |
| Current Version | 5 |

Change History

| Nr. | Date | Version | Changed Chapter | Type of Change | Author | State |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 03.12.25 | 1 | 2-5 |  | Aistleitner Daniel |  |
| 2 | 07.12.25 | 2 | 1-6 | renewal | Aistleitner Daniel |  |
| 3 | 10.12.2025 | 3 | 1-8 | finished | Aistleitner Daniel |  |
| 4 | 12.12.2025- | 4 | 1-8 | review | Brückl Matthias |  |
| 5 | 25.01.2026 | 5 | 1-8 | update | Brückl Matthias |  |

Table of Contents

1	Introduction	3

2	General	3

2.1	Aim and purpose of the document	3

2.2	Initial situation	3

2.3	Project Context	3

2.4 Abbreviations	3

2.5	Teams and Interfaces	4

3	Concept	4

3.1	Goal(s) of the provider specification	4

3.2	Specifications Goals and benefits for the user	4

3.3	Target group(s)	4

4	Functional Requirements	5

4.1	Requirement 1 – Sensor Data Acquisition	5

4.2	Requirement 2 – Data Processing and Storage	5

4.3	Requirement 3 – Web Interface	5

5	Non-Functional Requirements	5

5.1	Clear & easy-to-understand Web-Design	Fehler! Textmarke nicht definiert.

5.2	Legal Requirements	5

5.3	Technical Requirements	5

6	Framework conditions	6

6.1	Schedule	6

6.2	Technical Requirements	6

6.3	Problem Analysis	6

6.4	Quality Requirements	6

7	Delivery and Acceptance conditions	7

8	Appendix	7

# Introduction

This requirements specification describes all functional and non-functional requirements for the development of a weather station. It forms the binding foundation for planning and implementation of the project. It also explains the points on which the client, HTL Wels, and the contractor, Matthias Brückl, agreed.
The goal of the project is to design and implement a fully functional weather station capable of measuring environmental data such as temperature, humidity, air pressure, light intensity, UV index, and wind speed.
The system will be based on a Raspberry Pi Zero and various sensors. The collected data shall be processed locally and displayed via a web-based user interface, without the use of cloud services.
The final product will be energy-efficient and housed in a minimalist enclosure.

# General

## Aim and purpose of the document

The purpose of this document is to clearly define all requirements, constraints, and objectives of the Weather Station project.
It provides a common understanding for all project participants and serves as the basis for development.

## Initial situation

The project was initiated as part of a school assignment.

No existing weather station system is available. The entire software is being developed from scratch by the project team. The hardware is being purchased online.

The team already possesses basic knowledge of electronics, programming, and web development.

## Project Context

This project is an independent project and is not directly dependent on other projects. However, similar projects, for example “Indoor weather stations” or “Smart gardening systems”, may provide inspiration or technical know-how.

## 2.4 Abbreviations

| Abbreviation | Meaning |
| --- | --- |
| API | Application Programming Interface |
| DB | Database |
| UV | Ultraviolet |
| REST | Representational State Transfer |
| UI | User Interface |

## Teams and Interfaces

| Rolle(n) | Name | Telefon | E-Mail | Team |
| --- | --- | --- | --- | --- |
| Project-Owner | HTL Wels | 0123456789 | office@htl-wels.at | HTL Wels |
| Project-Manager | Matthias Brückl | 0123456789 | matthias.brückl@htl-wels.at | Wetterstation |
| Project Team Member | Daniel Aistleitner | 0123456789 | daniel.aistleitner@htl-wels.at | Wetterstation |
| Project Team Member | Klaus Bergbauer | 0123456789 | klaus.bergbauer@htl-wels.at | Wetterstation |
| Hardware Advisor | Andreas Wögerbauer | 0123456789 | andreas.woegerbauer@htl-wels.at | HTL Wels |
| Advisor | Susanne Loidl | 0123456789 | susanne.loidl@htl-wels.at | HTL Wels |

# Concept

## Goal(s) of the provider specification

- Build a fully functional weather station.

- The measurement of temperature, humidity, air pressure, UV and windspeed

- Implement a web-based data visualisation system.

- Create a minimalistic, aesthetically pleasing case.

- Learn new skills (sensor handling, electronics, full-stack development).

## Specifications Goals and benefits for the user

- Accurate measurement of environmental data in the garden

- A self-sustaining device with solar power.

- A live dashboard accessible via web browser.

- A durable and visually appealing design.

## Target group(s)

- Home users with interest in local weather conditions.

- Garden owners wanting real-time climate data.

- Technology enthusiasts seeking a simple, local, non-cloud solution.

# Functional Requirements

## Sensor Data Measurement

- Temperature

  - Range: -40°C to +80°C

  - Accuracy: ±0.5°C

  - Sampling Rate: Every 10 seconds

  - Output Format: Celsius with 1 decimal places

  - Error Handling: Display "N/A" if sensor fails, log error to system

- Humidity

  - Range: 0% to 100% relative humidity

  - Accuracy: ±3% relative humidity

  - Sampling Rate: Every 10 seconds

  - Output Format: Percentage with 1 decimal place

  - Alert: Notify if humidity >70% (mold risk) or <30% (too dry)

- Air pressure

  - Range: 300 hPa to 1100 hPa

  - Accuracy: ±1,5 hPa

  - Sampling Rate: Every 10 seconds

  - Output Format: Hectopascal (hPa) with 1 decimal place

- UV index

  - Range: 0 to 20+ mW/cm2

  - Accuracy: ±10 UV index

  - Sampling Rate: Every 10 seconds (daylight hours only)

  - Output Format: Integer value (0-15+)

- Wind speed (with pinwheel)

  - Range: 0 to 100 km/h

  - Accuracy: ±5%

  - Sampling Rate: Continuous with 10-second rolling average

  - Output Format: km/h with 1 decimal place

  - Gust Detection: Record maximum wind speed in 1-minute intervals

## Data Processing and Storage

- locale Database

  - Technology: use a lightweight SQLite

  - Data structure: Save time ISO 8601 format, a unique Sensor-ID and the maesurement value

  - Memory Space: Delete all data, which is older than 30 days, to have always enough space

- Update of the data

  - Interval: every second

- API

  - Architecture: a RESTful API to show the stored data in the web interface

  - Endpoints: GET /api/sensors/current” to get the current values and “GET /api/sensors/history” to get historical value of for example the last 24 hours

## Web Interface

- Visualization

  - Dashboard: show all important values at first sight

  - Latency: update the data every second

  - Mobile readiness: build a responsive layout to ensure usability on PC, Tablet and smartphone

  - visual indicators:use color coded status rings for example the UV index

| Green | Low | UX Index 1-2 |
| --- | --- | --- |
| Yellow | Moderate | UX Index 3-5 |
| Orange | High | UX Index 6-7 |
| Red | Very High | UX Index 8-10 |
| Purple | Extreme | UX Index 11+ |

- Development

  - Technology: Build Frontend with React Vite

  - Styling: use Tailwind fro clean design

- Network access

  - Connectivity: the web interface is accessible via a domain using mDNS for easy discovery without static IP addressesS

## Case

- Design

  - Style: “Dieter Rams Design”

  - Colors: White, Black, Creme, Grey

  - Material: 3D printed PETG

- functional Aspects

  - Ventilation: integration of vents to ensure accurate sensor readings

  - Mounting: Wall mounting and stand feet

  - Connections: recessed USB-C port

# Non-Functional Requirements

## General Requirements

- Modular and extendable system design

- Easy maintenance during project runtime

## Legal Requirements

- No personal data processing

- No cloud services used

## Technical Requirements

- Raspberry Pi Zero 2W

- Local database

- REST API

- Web frontend

# Framework conditions

## Schedule

- Project start: 12.01.2026

- Project end: 10.06.2026

Main phases:

- Hardware setup & sensor integration

- Backend (DB, API, server)

- Frontend (framework, design, integration)

- Power system (solar + battery)

- 3D case design and assembly

## Technical Requirements

Hardware Requirements

| Minimum | Recommended |
| --- | --- |
| Raspberry Pi Zero 2W | Raspberry Pi Zero 2W |
| BME280, DHT11, VEML6075 | BME280, DHT22, VEML6075 |
| Pinwheel for wind measurement | Pinwheel for wind measurement |
| --- | Solar panel + Li-ion battery |
| PETG filament for 3D printing | ABS filament for 3D printing |
| Breadboard and wiring | Breadboard and wiring |

Software Recommended Requirements

- Git for version control

- Visual Studio Code

- Raspberry Pi OS (Lite)

- Python 3

- React

- Sensor Libraries

- SQL

## Problem Analysis

| Risk | Solution |
| --- | --- |
| Sensor inaccuracies | Calibration and testing |
| Power instability | Battery buffering |
| Software bugs | Incremental testing |

## Quality Requirements

- Code reviews within team

- Component testing after each milestone

- 48-hour uninterrupted test run required

- Final acceptance by teacher

# Delivery and Acceptance conditions

The project is considered complete when:

- All sensors work reliably.

- Web dashboard displays all live data.

- Device runs power-autonomously via solar energy.

- Documentation is complete.

- Device is formally handed over to the teacher.

# Appendix

- Project Handbook

- Goals document

- Sensor datasheets

- Circuit drawings

- 3D case models

- API documentation